SIMATS ENGINEERING (SAVEETHA SCHOOL OF ENGINEERING)

Department of Computer Science and Engineering

ASSESSMENT TOOL 2 – Short Answers

| Course Code: | ITA03 | Course Title: | Mobile computing |
| --- | --- | --- | --- |
| CO Assessed: | CO1 | Assessment: | Assessment Tool 2 – Short Answers |
| Weightage: | 30 % of CIA | Total Marks: | 100 Marks |
| CO1 Statement: | Analyze the mobile network components, mobile base station, mobile infrastructures and protocols in cellular systems. |  |  |
| Student Name: | Alheena yusra A | Reg. No.: | 192424026 |
| Section / Batch: | BTech -AIDS | Date: | 12-10-2026 |

Code of Conduct I ALHEENA YUSRA A (192424026) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the Student: _________________________

Instructions - This test contains 50 questions, Total: 100 Marks. - Duration: 1 hour.

| Faculty In-charge | Course Coordinator | Head of Department |
| --- | --- | --- |
| Signature: _______________ | Signature: _______________ | Signature: _______________ |
| Date: ___________________ | Date: ___________________ | Date: ___________________ |

## Q1. A field engineer downloads a 15 MB firmware file on a mobile link that sustains an average of 12 Mbps. How long will the download take?

Formula: Time = File size (bits) / Data rate

Calculation:

1. File size = 15 x 8 = 120 megabits

2. Time = 120 / 12 = 10 seconds

Answer: 10 seconds.

Explanation: Convert megabytes to megabits by multiplying by 8, then divide by the download speed.

## Q2. A background sync service on a smartphone draws an average current of 160 mA from a 4000 mAh battery. How many hours can the phone run this service before the battery is empty?

Formula: Battery life = Battery capacity / Current consumption

Calculation:

1. Battery life = 4000 mAh / 160 mA = 25 hours

Answer: 25 hours.

Explanation: This assumes constant current consumption and an ideal battery.

## Q3. A handset sends a 1500-byte packet over a 2 Mbps uplink, and the propagation delay to the base station is 5 ms. What is the total one-way delay for the packet?

Formula: Total delay = Transmission delay + Propagation delay

Calculation:

1. Packet size = 1500 x 8 = 12,000 bits

2. Transmission delay = 12,000 / 2,000,000 = 0.006 s = 6 ms

3. Total delay = 6 + 5 = 11 ms

Answer: 11 ms.

Explanation: Add the time to transmit the packet to the propagation delay.

## Q4. A mobile app compresses a 4 MB photo by 75% before uploading it over a 400 kbps packet-data link. How long does the upload take?

Formula: Upload time = Compressed file size (bits) / Data rate

Calculation:

1. Remaining size = 4 x (100 - 75) / 100 = 1 MB

2. Size in bits = 1 x 8 = 8 megabits

3. Time = 8,000,000 / 400,000 = 20 seconds

Answer: 20 seconds.

Explanation: A 75% reduction leaves 25% of the original file.

## Q5. A planner locates co-channel cells by moving i = 2 cells along one axis and then j = 1 cell after a 60-degree turn. What cluster size N does this produce?

Formula: N = i^2 + i*j + j^2

Calculation: N = 2^2 + (2 x 1) + 1^2 = 4 + 2 + 1 = 7

Answer: N = 7 cells.

Explanation: Substitute the shift values i = 2 and j = 1 into the cluster-size formula.

## Q6. An operator adopts a 12-cell reuse cluster. What is the co-channel reuse ratio Q = D/R for this design?

Formula: Q = D/R = square root of (3N)

Calculation: Q = square root of (3 x 12) = square root of 36 = 6

Answer: Q = 6.

Explanation: The reuse ratio is the co-channel distance divided by the cell radius. Note: N = 12 is not a standard hexagonal cluster size under the usual integer-shift formula; this uses the requested formula.

## Q7. Cells in a 7-cell cluster have a radius of 2 km. What is the minimum distance between the centres of two co-channel cells?

Formula: D = R x square root of (3N)

Calculation: D = 2 x square root of (3 x 7) = 2 x square root of 21 = 2 x 4.583 = 9.17 km

Answer: Approximately 9.17 km.

Explanation: Multiply the cell radius by the square root of three times the cluster size.

## Q8. A district of 1000 square km is to be covered with hexagonal cells of 1 km radius. Approximately how many cells are needed?

Formula: Hexagonal cell area = (3 x square root of 3 / 2) x R^2

Calculation:

1. Cell area = (3 x square root of 3 / 2) x 1^2 = 2.598 square km

2. Number of cells = 1000 / 2.598 = 384.9

Answer: Approximately 385 cells.

Explanation: Divide the total area by the area of one ideal hexagonal cell.

## Q9. Each cell must support 50 full-duplex channels, each using 25 kHz in each direction, with a 7-cell reuse pattern. How much total spectrum does the operator need?

Formula: Spectrum per direction = Channels per cell x Cluster size x Bandwidth per channel

Calculation: 50 x 7 x 25 kHz = 8750 kHz = 8.75 MHz per direction

Answer: 8.75 MHz per direction, or 17.5 MHz for both directions combined.

Explanation: This assumes separate spectrum blocks for the two directions.

## Q10. A town is served by 10 cells. Traffic growth forces every cell to be split into microcells of half the original radius. How many cells will cover the same area after splitting?

Formula: New cells = Original cells x (Old radius / New radius)^2

Calculation: 10 x (2)^2 = 10 x 4 = 40

Answer: 40 cells.

Explanation: Halving the radius makes each cell cover one-quarter of the original area.

## Q11. A cell with 60 channels is converted to three 120-degree sectors, with channels shared equally. How many channels does each sector get?

Formula: Channels per sector = Total channels / Number of sectors

Calculation: 60 / 3 = 20

Answer: 20 channels per sector.

Explanation: Divide the available channels equally among the three sectors.

## Q12. A system has 1000 duplex channels and adopts a 4-cell cluster. How many channels does each cell receive?

Formula: Channels per cell = Total channels / Cluster size

Calculation: 1000 / 4 = 250

Answer: 250 channels per cell.

Explanation: Divide the channel pool equally among the four cells.

## Q13. A 7-cell reuse system operates where the path-loss exponent is 4. Considering only the six first-tier co-channel interferers, estimate the signal-to-interference ratio in dB.

Formula: S/I = (D/R)^n / 6, where D/R = square root of (3N)

Calculation:

1. D/R = square root of (3 x 7) = square root of 21

2. S/I = (square root of 21)^4 / 6 = 441 / 6 = 73.5

3. S/I in dB = 10 x log10(73.5) = 18.66 dB

Answer: Approximately 18.66 dB.

Explanation: This assumes six equally distant first-tier interferers and ignores noise.

## Q14. A designer tests whether a 4-cell cluster can meet an 18 dB S/I target with a path-loss exponent of 4 and six first-tier interferers. What S/I does N = 4 actually give, and is the target met?

Formula: S/I = (square root of (3N))^n / 6

Calculation:

1. D/R = square root of (3 x 4) = square root of 12

2. S/I = (square root of 12)^4 / 6 = 144 / 6 = 24

3. S/I in dB = 10 x log10(24) = 13.80 dB

Answer: 13.80 dB; the 18 dB target is not met.

Explanation: The calculated S/I is lower than the required target.

## Q15. A drive test records a serving signal of -70 dBm, co-channel interference of -88 dBm and a noise floor of -100 dBm. Compute the SINR in dB.

Formula: SINR = Signal power / (Interference power + Noise power)

Calculation:

1. Interference relative to signal = 10^((-88 + 70)/10) = 0.01585

2. Noise relative to signal = 10^((-100 + 70)/10) = 0.001

3. SINR = 10 x log10(1 / (0.01585 + 0.001)) = 17.73 dB

Answer: Approximately 17.73 dB.

Explanation: Convert dBm values to relative linear powers before adding interference and noise.

## Q16. A 1800 MHz carrier is used for a GSM rollout. What is its wavelength?

Formula: Wavelength = Speed of light / Frequency

Calculation: (3 x 10^8) / (1.8 x 10^9) = 0.1667 m

Answer: 0.167 m, or 16.67 cm.

Explanation: Wavelength is the speed of light divided by the carrier frequency.

## Q17. At 900 MHz, at what distance does the free-space path loss reach 120 dB?

Formula: FSPL = 32.44 + 20 log10(f in MHz) + 20 log10(d in km)

Calculation:

1. 120 = 32.44 + 20 log10(900) + 20 log10(d)

2. 20 log10(900) = 59.08

3. 20 log10(d) = 120 - 32.44 - 59.08 = 28.48

4. d = 10^(28.48/20) = 26.55 km

Answer: Approximately 26.55 km.

Explanation: Rearrange the free-space path-loss formula to solve for distance.

## Q18. The received power at a 1 km reference distance is -50 dBm in an urban area with a path-loss exponent of 3.5. What power is expected at 4 km?

Formula: Pr(d) = Pr(d0) - 10n log10(d/d0)

Calculation: Pr(4) = -50 - 10 x 3.5 x log10(4) = -50 - 21.07 = -71.07 dBm

Answer: Approximately -71.07 dBm.

Explanation: Received power decreases as distance increases due to path loss.

## Q19. A 2100 MHz receiver can tolerate a maximum Doppler shift of 150 Hz. What is the highest vehicle speed, in km/h, the link can support?

Formula: Speed = Doppler shift x Speed of light / Carrier frequency

Calculation: v = (150 x 3 x 10^8) / (2.1 x 10^9) = 21.43 m/s

Convert: 21.43 x 3.6 = 77.14 km/h

Answer: Approximately 77.14 km/h.

Explanation: Assumes motion directly toward or away from the transmitter.

## Q20. A mobile channel has a maximum Doppler shift of 200 Hz. Using T = 0.423 / f(d), estimate the channel's coherence time.

Formula: Coherence time = 0.423 / Doppler shift

Calculation: T = 0.423 / 200 = 0.002115 seconds = 2.115 ms

Answer: 2.115 ms.

Explanation: Coherence time estimates how long the wireless channel remains approximately unchanged.

## Q21. A GSM operator has 25 MHz in each direction, uses 200 kHz carriers and leaves one carrier as a guard. Each carrier carries 8 time slots. How many traffic channels are available?

Formula: Traffic channels = Usable carriers x Time slots per carrier

Calculation:

1. Total carriers = 25 MHz / 0.2 MHz = 125

2. Usable carriers = 125 - 1 = 124

3. Traffic channels = 124 x 8 = 992

Answer: 992 time-slot channels.

Explanation: Assumes all remaining time slots are used for traffic; real networks reserve some for control signalling.

## Q22. A GSM burst period lasts 576.9 microseconds and the gross channel bit rate is 270.833 kbps. How many bits fit in one burst period?

Formula: Bits = Bit rate x Time

Calculation: 270,833 bits/s x 576.9 x 10^-6 s = 156.25 bits

Answer: Approximately 156 bits.

Explanation: Multiply the bit rate by the burst duration.

## Q23. An IS-95 CDMA link spreads a 9.6 kbps voice stream at a chip rate of 1.2288 Mcps. What is the processing gain in dB?

Formula: Processing gain = 10 log10(Chip rate / Data rate)

Calculation: 10 log10(1,228,800 / 9600) = 10 log10(128) = 21.07 dB

Answer: Approximately 21.07 dB.

Explanation: Processing gain compares the chip rate with the original data rate.

## Q24. An FDMA system has 12.5 MHz of spectrum, 30 kHz channels and a 10 kHz guard band at each edge of the allocation. How many channels can it provide?

Formula: Channels = Usable bandwidth / Channel bandwidth

Calculation:

1. Guard bandwidth = 10 + 10 = 20 kHz

2. Usable bandwidth = 12,500 - 20 = 12,480 kHz

3. Channels = 12,480 / 30 = 416

Answer: 416 channels.

Explanation: Remove both guard bands before dividing by the channel bandwidth.

## Q25. An LTE carrier has 1200 usable subcarriers spaced 15 kHz apart, grouped 12 per resource block. How many resource blocks does the carrier offer?

Formula: Resource blocks = Total subcarriers / Subcarriers per resource block

Calculation: 1200 / 12 = 100

Answer: 100 resource blocks.

Explanation: Each resource block contains 12 subcarriers.

## Q26. Handsets contend on a slotted-ALOHA random access channel with an offered load of G = 0.5 attempts per slot. What throughput does the channel achieve?

Formula: Throughput S = G x e^(-G)

Calculation: S = 0.5 x e^(-0.5) = 0.5 x 0.6065 = 0.3033

Answer: 0.3033 successful transmissions per slot.

Explanation: This is the standard slotted-ALOHA throughput formula.

## Q27. A location area has 25 cells, and the network must page 2000 incoming calls per hour in every cell of that area. How many paging messages are sent per hour?

Formula: Total messages = Number of cells x Messages per cell

Calculation: 25 x 2000 = 50,000

Answer: 50,000 paging messages per hour.

Explanation: Each cell sends 2000 messages per hour.

## Q28. A BSC controls 40 BTSs. Each BTS has 3 sectors, each sector has 2 TRXs, and each TRX carries 8 time slots. How many time slots does the BSC manage?

Formula: Total time slots = BTSs x Sectors x TRXs x Slots

Calculation: 40 x 3 x 2 x 8 = 1920

Answer: 1920 time slots.

Explanation: Multiply the number of units at each network level.

## Q29. One MSC can serve 500,000 subscribers. How many MSCs does a city with 2.4 million subscribers need?

Formula: MSCs = Total subscribers / Capacity per MSC

Calculation: 2,400,000 / 500,000 = 4.8; round up to 5

Answer: 5 MSCs.

Explanation: Round up so that all subscribers can be served.

## Q30. An Abis link runs on an E1 line of 2.048 Mbps, divided into 16 kbps sub-channels. How many sub-channels does the link carry?

Formula: Sub-channels = Total rate / Rate per sub-channel

Calculation: 2048 kbps / 16 kbps = 128

Answer: 128 theoretical sub-channels.

Explanation: Actual E1 framing and Abis configuration may reduce usable capacity.

## Q31. An HLR stores 1.5 KB of profile data per subscriber. How much storage is needed for 8 million subscribers?

Formula: Total storage = Data per subscriber x Number of subscribers

Calculation: 1.5 KB x 8,000,000 = 12,000,000 KB = 12 GB (decimal units)

Answer: 12 GB.

Explanation: This excludes database indexes, replication and other overhead.

## Q32. A VLR serves 60,000 visiting subscribers, and each performs 4 location updates per hour. How many location updates per second must the VLR process?

Formula: Updates per second = Hourly updates / 3600

Calculation:

1. Hourly updates = 60,000 x 4 = 240,000

2. Updates per second = 240,000 / 3600 = 66.67

Answer: Approximately 66.67 updates per second.

Explanation: There are 3600 seconds in one hour.

## Q33. A GSM full-rate speech codec outputs 13 kbps, and channel coding raises this to 22.8 kbps over the air. What percentage overhead does channel coding add?

Formula: Overhead % = (Final rate - Original rate) / Original rate x 100

Calculation: ((22.8 - 13) / 13) x 100 = (9.8 / 13) x 100 = 75.38%

Answer: Approximately 75.38%.

Explanation: The added bits provide error detection and correction.

## Q34. A 1400-byte voice payload is carried with a 20-byte IP header, an 8-byte UDP header and a 12-byte RTP header. What is the protocol efficiency?

Formula: Efficiency = Payload / Total packet size x 100

Calculation:

1. Headers = 20 + 8 + 12 = 40 bytes

2. Total size = 1400 + 40 = 1440 bytes

3. Efficiency = (1400 / 1440) x 100 = 97.22%

Answer: 97.22%.

Explanation: Efficiency is the percentage of the packet that contains voice payload.

## Q35. An AMR codec produces a 244-bit speech frame every 20 ms, and each frame is sent with 40 bytes of IP/UDP/RTP headers. What bit rate does the stream use on the link?

Formula: Bit rate = Total bits per frame / Frame duration

Calculation:

1. Header bits = 40 x 8 = 320 bits

2. Total bits = 244 + 320 = 564 bits

3. Duration = 20 ms = 0.02 s

4. Bit rate = 564 / 0.02 = 28,200 bits/s = 28.2 kbps

Answer: 28.2 kbps.

Explanation: Include speech and header bits.

## Q36. In the packet core, a 1000-byte user packet is tunnelled with an 8-byte GTP-U header, an 8-byte UDP header and a 20-byte IP header. What percentage of the transmitted bytes is tunnelling overhead?

Formula: Overhead % = Header bytes / Total transmitted bytes x 100

Calculation:

1. Header bytes = 8 + 8 + 20 = 36

2. Total bytes = 1000 + 36 = 1036

3. Overhead = (36 / 1036) x 100 = 3.47%

Answer: Approximately 3.47%.

Explanation: Divide the extra header bytes by the complete packet size.

## Q37. A GSM LAPDm signalling frame is 23 bytes long, of which 3 bytes are header. What is the frame's payload efficiency?

Formula: Efficiency = Payload / Total frame size x 100

Calculation:

1. Payload = 23 - 3 = 20 bytes

2. Efficiency = (20 / 23) x 100 = 86.96%

Answer: Approximately 86.96%.

Explanation: The remaining 20 bytes carry payload.

## Q38. A PLMN uses a 15-digit IMSI with a 3-digit MCC and a 2-digit MNC. How many digits are left for the MSIN, and how many subscribers can the operator number?

Formula: MSIN digits = IMSI digits - MCC digits - MNC digits

Calculation: 15 - 3 - 2 = 10 digits; possible combinations = 10^10

Answer: 10 MSIN digits and up to 10 billion possible identities.

Explanation: This is a theoretical maximum assuming all combinations are available.

## Q39. Another operator in the same country is assigned a 3-digit MNC. How many unique subscriber identities can it issue?

Formula: MSIN digits = IMSI digits - MCC digits - MNC digits

Calculation: 15 - 3 - 3 = 9 digits; possible combinations = 10^9

Answer: Up to 1 billion possible identities.

Explanation: A 3-digit MNC leaves 9 digits for the MSIN.

## Q40. A core-network packet gateway handles 40 Gbps. In the busy hour, each user averages 2 Mbps when active and is active 10% of the time. How many users can the gateway support?

Formula: Average rate per user = Active rate x Activity factor

Calculation:

1. Gateway capacity = 40 Gbps = 40,000 Mbps

2. Average user rate = 2 x 0.10 = 0.2 Mbps

3. Users = 40,000 / 0.2 = 200,000

Answer: 200,000 users, theoretically.

Explanation: This estimate excludes protocol overhead and reserved capacity.

## Q41. On a data session, one-way delays are 10 ms on the radio link, 5 ms on backhaul, 3 ms in the core and 22 ms across the internet. What is the round-trip time?

Formula: RTT = 2 x Total one-way delay

Calculation: One-way delay = 10 + 5 + 3 + 22 = 40 ms; RTT = 2 x 40 = 80 ms

Answer: 80 ms.

Explanation: Assumes the return path has the same delay.

## Q42. A network starts handoff when the received signal falls to -95 dBm, and the minimum usable signal for voice is -100 dBm. What is the handoff margin?

Formula: Margin = Handoff threshold - Minimum usable signal

Calculation: -95 - (-100) = -95 + 100 = 5 dB

Answer: 5 dB.

Explanation: Handoff starts 5 dB before the signal reaches the minimum usable level.

## Q43. A vehicle travels at 72 km/h along a highway lined with cells 2 km across. How many handoffs does it make in one hour?

Formula: Handoffs per hour = Distance travelled / Cell width

Calculation: Distance in one hour = 72 km; handoffs = 72 / 2 = 36

Answer: Approximately 36 handoffs per hour.

Explanation: Assumes a straight route through regularly spaced cells.

## Q44. A handoff margin of 6 dB is set. The signal fades by 0.1 dB per metre near the cell edge, and the user moves at 15 m/s. How much time does the network have to complete the handoff?

Formula: Time = Available distance / Speed

Calculation: Distance = 6 / 0.1 = 60 m; time = 60 / 15 = 4 seconds

Answer: 4 seconds.

Explanation: The margin corresponds to 60 metres under the given fading rate.

## Q45. A serving cell is received at -85 dBm and a neighbour at -82 dBm, with a 4 dB handoff hysteresis. Will handoff be triggered? If not, how much stronger must the neighbour become?

Formula: Neighbour advantage = Neighbour signal - Serving signal

Calculation: -82 - (-85) = 3 dB; remaining difference = 4 - 3 = 1 dB

Answer: No. The neighbour must become another 1 dB stronger relative to the serving cell.

Explanation: The current 3 dB advantage is below the 4 dB hysteresis threshold.

## Q46. One million inbound roamers each make 2 location updates per day. Each update exchanges 5 signalling messages of 200 bytes. How much roaming signalling traffic is generated per day?

Formula: Traffic = Roamers x Updates per day x Messages per update x Bytes per message

Calculation: 1,000,000 x 2 x 5 x 200 = 2,000,000,000 bytes = 2 GB (decimal units)

Answer: 2 GB per day.

Explanation: This excludes additional transport overhead.

## Q47. A user crosses 12 cell boundaries per hour, and 20% of those crossings are also location-area boundaries. How many location updates does the user trigger per hour?

Formula: Updates = Cell crossings x Location-area boundary percentage

Calculation: 12 x 0.20 = 2.4

Answer: An average of 2.4 updates per hour.

Explanation: This is an average; actual individual counts are whole numbers.

## Q48. A cell serves 2000 subscribers. Each makes 3 calls in the busy hour, and calls last 2 minutes on average. What traffic, in Erlangs, does the cell carry?

Formula: Traffic (Erlangs) = Total call minutes / 60

Calculation: Total call minutes = 2000 x 3 x 2 = 12,000 minutes; traffic = 12,000 / 60 = 200 Erlangs

Answer: 200 Erlangs.

Explanation: One Erlang represents one hour of continuous occupancy during the measurement period.

## Q49. Network statistics show 900 dropped calls out of 45,000 established calls in a day. What is the call drop rate?

Formula: Drop rate = Dropped calls / Established calls x 100

Calculation: (900 / 45,000) x 100 = 2%

Answer: 2%.

Explanation: Divide dropped calls by established calls and multiply by 100.

## Q50. A cell with 24 channels is offered 18 Erlangs at a 2% grade of service. How much traffic is actually carried, and what is the channel utilisation?

Formula 1: Carried traffic = Offered traffic x (1 - Blocking probability)

Formula 2: Utilisation = Carried traffic / Number of channels x 100

Calculation:

1. Blocking probability = 2% = 0.02

2. Carried traffic = 18 x (1 - 0.02) = 17.64 Erlangs

3. Utilisation = (17.64 / 24) x 100 = 73.5%

Answer: 17.64 Erlangs carried and 73.5% utilisation under the simplified assumption.

Explanation: This assumes 2% of offered traffic is blocked. Exact carried traffic should be calculated using Erlang B for 24 channels and 18 Erlangs.